SINGLE AND TWO LAYER NEURAL NETWORK

DYNAMICS –

Consider the two-link dynamic system

M∅∅+C∅,∅+G∅+τd∅,∅= τ

Where ∅≜∅1∅2, ∅≜∅1∅2,  ∅≜∅1∅2, τ≜τ1τ2 ϵ R2 are the angle, angular velocity, angular acceleration, and input torque of the arm joints.

M∅≜m1l12+m2l2+2l1l2c2+l22m2l1l2c2+l22m2l1l2c2+l22m2l22 ϵ R2×2

C∅,∅=-2m2l1l2s2∅1∅20+-m2l1l2s2∅22m2l1l2s2∅12 ϵ R2,

G∅=m1+m2gl1c1+m2gl2c12m2gl2c12 ϵ R2,

where mi,li ϵ R>0  are the unknow constant mass and length of the link i and gravity is g = 9.8 m/s^2 and,

ci= cos(∅i),

si= sin(∅i),

c12= cos(∅1+ ∅2),

s12= sin(∅1+∅2),

And τd∅,∅ ϵ R2 is the unknown, unstructured state-dependent disturbance that is a function of the angles and angular velocity.

Assuming that m ≤mi≤m, l ≤li≤l are known bounds on the unknown mass and length of link i, with mi=2kg, li=0.5m, m=1kg, m=3kg, l=0.25m, l=0.25m. Also, assuming that ∅,∅,∅ are all measurable.

Let the desired trajectory have a magnitude of ∅d= ∅d1∅d2=π8π4, at a frequency of f∅d≜0.2Hz, a phase shift a∅d≜ π2, and bias of b∅d≜ π2π4 , such that ∅d1(t)∅d2(t)≜∅d1sin(2πf∅dt-a∅d)- b∅d∅d2sin(2πf∅dt-a∅d)+ b∅d which implies ∅d1(t)∅d2(t)≜∅d1(2πf∅d)cos(2πf∅dt-a∅d)∅d2(2πf∅d)cos(2πf∅dt-a∅d) and ∅d1(t)∅d2(t)≜-2πf∅d2∅d1sin(2πf∅dt-a∅d)-2πf∅d2∅d2sin(2πf∅dt-a∅d).

Designing a controller under the assumption that the structure is completely unknown for τd∅,∅

(1) Robust Controller

From the dynamics,

M∅∅+C∅,∅+G∅+τd∅,∅= τ

M∅∅+C∅,∅+G∅ accounts to the structured uncertainties and τd∅,∅ represents the unstructured uncertainties. And as a result, we can design a linear in the parameter design for the structured uncertainties like the concurrent learning and integral concurrent learning design. And the unstructured uncertainties can be estimated using a single layer neural network.

For a single layer neural network  τd∅,∅ can be estimated as

τd∅,∅= WTσ∅,∅+ε∅,∅

Where Wϵ RL×2  is the output weights, σ(.)ϵ RL is the output basis functions, and the reconstruction error ε∅,∅ ϵ R2.

Error Dynamics-

e=∅d-∅

e= ∅d-∅

e= ∅d-∅

Reference tracking error

r= e+ αe

r= e+αe

r=∅d-∅+αe

Multiplying both sides by M∅ we get,

M(∅)r=M∅∅d-M(∅)∅+M∅αe

From the dynamics we know,

M∅∅=-C∅,∅-G∅-τd∅,∅+ τ

On substitution we get,

M(∅)r=M∅∅d-M(∅)∅+M∅αe

M∅r=M∅∅d-(-C∅,∅-G∅-τd∅,∅+ τ)+M∅αe

Add and subtract by 12M∅,∅r

M(∅)r=M∅∅d+C∅,∅+G∅+τd∅,∅- τ+M∅αe ±12M∅,∅r

M(∅)r=M∅(∅d+αe)+C∅,∅+G∅+τd∅,∅- τ ±12M∅,∅r

We can estimate M∅ (∅d+αe)+C∅,∅+G∅+12M∅,∅r=Yθ since it is linear in the unknown parameters and we were able to develop the unknown parameters θ as follows,

θ1θ2θ3θ4θ5=m1l12+m2l12+m2l22m2l1l2m2l12m1+m2l1m2l2

And Y= YM∅,∅d+αe+Yc∅,∅+YG∅+12YM∅,∅, r

Now,

M∅r=Yθ+τd∅,∅-τ-12M∅,∅r

Let the Lyapunov Candidate be,

V= 12eTe+ 12rT M∅r+12θTΓθ-1θ

V=eTe+12rT Mr+rT M∅r+θTΓθ-1θ

We know,

e=r- αe

Mr=Yθ+τd-τ-12Mr

θ=-θ

V=eT(r- αe)+12rT Mr+rT (Yθ+τd-τ-12Mr)+θTΓθ-1θ

On simplification we get,

V=eT(r- αe)+rT (Yθ+τd-τ)+θTΓθ-1θ

Input design,

τ=e+Yθ+βrr+βτdsgn(r)

On substitution we get,

V=- eTαe+rTYθ+rTτd-rTβrr-rTβτdsgnr+θTΓθ-1θ

We can introduce an inequality since, rTτd-rTβτdsgnr≤0 if βτd>τd=60Nm

Now,

V≤- eTαe+rTYθ-rTβrr-θTΓθ-1θ

Design Parametric error θ= ΓθYTr

We get,

V≤- eTαe+rTYθ-rTβrr-θTΓθ-1ΓθYTr

V≤- eTαe-rTβrr

Using Barbalat’s Lemma we can show the e,ryields 0 which implies Asymptotic tracking

(2) Single Layer Neural Network Design

From the dynamics,

M∅∅+C∅,∅+G∅+τd∅,∅= τ

M∅∅+C∅,∅+G∅ accounts to the structured uncertainties and τd∅,∅ represents the unstructured uncertainties. And as a result, we can design a linear in the parameter design for the structured uncertainties like the concurrent learning and integral concurrent learning design. And the unstructured uncertainties can be estimated using a single layer neural network.

For a single layer neural network  τd∅,∅ can be estimated as

τd∅,∅= WTσ∅,∅+ε∅,∅

Where Wϵ RL×2  is the output weights, σ(.)ϵ RL is the output basis functions, and the reconstruction error ε∅,∅ ϵ R2.

Error Dynamics-

e=∅d-∅

e= ∅d-∅

e= ∅d-∅

Reference tracking error

r= e+ αe

r= e+αe

r=∅d-∅+αe

Multiplying both sides by M∅ we get,

M(∅)r=M∅∅d-M(∅)∅+M∅αe

From the dynamics we know,

M∅∅=-C∅,∅-G∅-τd∅,∅+ τ

On substitution we get,

M(∅)r=M∅∅d-M(∅)∅+M∅αe

M∅r=M∅∅d-(-C∅,∅-G∅-τd∅,∅+ τ)+M∅αe

Add and subtract by 12M∅,∅r

M(∅)r=M∅∅d+C∅,∅+G∅+τd∅,∅- τ+M∅αe ±12M∅,∅r

M(∅)r=M∅(∅d+αe)+C∅,∅+G∅+τd∅,∅- τ ±12M∅,∅r

We can estimate M∅ (∅d+αe)+C∅,∅+G∅+12M∅,∅r=Yθ since it is linear in the unknown parameters and we were able to develop the unknown parameters θ as follows,

θ1θ2θ3θ4θ5=m1l12+m2l12+m2l22m2l1l2m2l12m1+m2l1m2l2

And Y= YM∅,∅d+αe+Yc∅,∅+YG∅+12YM∅,∅, r

The unstructured state-dependent disturbance τd∅,∅ will be estimated using the neural network.

Now,

M(∅)r=Yθ+WTσ∅,∅+ε∅,∅-τ-12M∅,∅r

Mr=Yθ+WTσ+ε-τ-12Mr

Let the estimated structured estimation error θ=θ-θ where  θϵR5

And unstructured estimation error W=W-W where θϵRL×2

Let the stacked errors

ξ= erθvec(W) ϵ R2+2+5+2L

Where,

vecW=W11⋮WL1W12⋮WL2

Let the Lyapunov candidate V be,

V= 12eTe+ 12rT M∅r+12θTΓθ-1θ+ 12tr(WTΓW-1W)

On differentiating we get,

V=eTe+12rT Mr+rT M∅r+θTΓθ-1θ+tr(WTΓW-1W)

Using 
 e=r- αe

Mr=Yθ+WTσ+ε-τ-12Mr

θ=-θ

W=-W

We get,

V=eTr- αe+12rTYθ+WTσ+ε-τ-12Mr+rT M∅r-θTΓθ-1θ+tr(-WTΓW-1W)

Designing Input τ

τ=Yθ+ WTσ+ βεsgnr+βrr+e

Where

Yθ+ WTσ=feedforward part of the input

βεsgnr+βrr+e=robust feedback part of the input

V=eTr- αe+12rTYθ+WTσ+ε-(Yθ+ WTσ+ βεsgnr+βrr+e)-12Mr+rT M∅r-θTΓθ-1θ+tr(-WTΓW-1W)

V=eTr-eTαe+12rTYθ+WTσ+ε-(Yθ+ WTσ+ βεsgnr+βrr+e)-12Mr+rT M∅r-θTΓθ-1θ+tr(-WTΓW-1W)

Since we know,

Yθ- Yθ=Yθ

WTσ- WTσ=WTσ

The above equation can be simplified to

V=-eTαe-rTβrr+rTε-rTβεsgnr+rTYθ+rTWTσ-θTΓθ-1θ+tr(-WTΓW-1W)

Choosing βε>ε where ε represents the upper bound on ε, we can say

rTε-βεr<0

Therefore, the above equation can be lower bounded as follows

V≤-eTαe-rTβrr+rTYθ+rTWTσ-θTΓθ-1θ+tr(-WTΓW-1W)

And form the property of trace

trbaT= aTb

rTWTσ=tr(WTσrT)

V≤-eTαe-rTβrr+rTYθ+tr(WTσrT)-θTΓθ-1θ+tr(-WTΓW-1W)

V≤-eTαe-rTβrr+rTYθ-θTΓθ-1θ+tr(-WTΓW-1W+WTσrT)

Designing structured estimate θ= ΓθYTr

And unstructured estimate W=ΓWσrT

On substitution,

V≤-eTαe-rTβrr+rTYθ-θTΓθ-1(ΓθYTr)+tr(-WTΓW-1(ΓWσr)+WTσrT)

V≤-eTαe-rTβrr+rTYθ-θTYTr+tr(-WTσrT+WTσrT)

Since rTYθ=θTYTr

V≤-eTαe-rTβrr+tr(-WTσrT+WTσrT)

V≤-eTαe-rTβrr

Using Barbalat’s Lemma we can show the e,ryields 0 which implies Asymptotic tracking

(3) Two-layer Neural Network

From the dynamics,

M∅∅+C∅,∅+G∅+τd∅,∅= τ

M∅∅+C∅,∅+G∅ accounts to the structured uncertainties and τd∅,∅ represents the unstructured uncertainties. And as a result, we can design a linear in the parameter design for the structured uncertainties like the concurrent learning and integral concurrent learning design. And the unstructured uncertainties can be estimated using a two-layer neural network.

For a two-layer neural network  τd∅,∅ can be estimated as

τd∅,∅= WTσΦ(ζ+ε∅,∅

Where Wϵ RL×2  is the output weights, σ(.)ϵ RL is the activation function,  Φ=VTζ is the inner features which has the input and inner weights where V is the inner weights and ζ is the input and the reconstruction error is ε∅,∅ ϵ R2.

Error Dynamics-

e=∅d-∅

e= ∅d-∅

e= ∅d-∅

Reference tracking error

r= e+ αe

r= e+αe

r=∅d-∅+αe

Multiplying both sides by M∅ we get,

M(∅)r=M∅∅d-M(∅)∅+M∅αe

From the dynamics we know,

M∅∅=-C∅,∅-G∅-τd∅,∅+ τ

On substitution we get,

M(∅)r=M∅∅d-M(∅)∅+M∅αe

M∅r=M∅∅d-(-C∅,∅-G∅-τd∅,∅+ τ)+M∅αe

Add and subtract by 12M∅,∅r

M(∅)r=M∅∅d+C∅,∅+G∅+τd∅,∅- τ+M∅αe ±12M∅,∅r

M(∅)r=M∅(∅d+αe)+C∅,∅+G∅+τd∅,∅- τ ±12M∅,∅r

We can estimate M∅ (∅d+αe)+C∅,∅+G∅+12M∅,∅r=Yθ since it is linear in the unknown parameters and we were able to develop the unknown parameters θ as follows,

θ1θ2θ3θ4θ5=m1l12+m2l12+m2l22m2l1l2m2l12m1+m2l1m2l2

And Y= YM∅,∅d+αe+Yc∅,∅+YG∅+12YM∅,∅, r

The unstructured state-dependent disturbance τd∅,∅ will be estimated using a two-layer neural network.

τd∅,∅= WTσΦ(ζ+ε∅,∅

Now,

M(∅)r=Yθ+WTσΦ(ζ)+ε∅,∅-τ-12M∅,∅r

Approximation for

σΦ= σΦ+ dσdΦΦ+εσ(Φ2)

Where,

Φ=Φ- Φ= VTζ-VTζ= Vζ

σ= σ+ dσdΦΦ+εσ

Now on substitution we get,

M(∅)r=Yθ+WT(σΦ+ dσdΦΦ+εσ(Φ2))+ε∅,∅-τ-12M∅,∅r

Add and subtract  WTσ'Φ

M∅r=Yθ+WTσΦ+ dσdΦΦ+εσΦ2+ε∅,∅-τ-12M∅,∅r±WTσ'Φ

M∅r=Yθ+WTσΦ-τ-12M∅,∅r+WTσ'Φ+ WTσ'Φ+WTεσΦ2+ε∅,∅

Let

WTσ'Φ+WTεσΦ2+ε∅,∅= δ

M∅r=Yθ+WTσΦ-τ-12M∅,∅r+WTσ'Φ+δ

Let the stacked errors

ξ= erθvec(W)vec(V) ϵ R2+2+5+2L+5l

Where,

vecW=W11⋮WL1W12⋮WL2  and  vecV= V11⋮V51⋮V1l⋮V5l

Lyapunov Candidate,

V= 12eTe+12rTM∅r+12θTΓθ-1θ+12tr(WTΓW-1W)+12tr(VTΓV-1V)

V=eTe+12rTM∅r+rTMr+θTΓθ-1θ+trWTΓW-1W+tr(VTΓV-1V)

We know,

r= e= r-αe

θ=-θ

W=-W

V=-V

And 
M∅r=Yθ+WTσΦ-τ-12M∅,∅r+WTσ'Φ+δ

On substitution we get,

V=eTe+12rTM∅r+rTMr+θTΓθ-1θ+trWTΓW-1W+tr(VTΓV-1V)

V=eTe+12rTM∅r+rT(Yθ+WTσΦ-τ-12M∅,∅r+WTσ'Φ+δ )+θTΓθ-1θ+trWTΓW-1W+tr(VTΓV-1V)

On simplification we get,

V=eTr-αe+rTYθ+WTσΦ-τ+WTσ'Φ+δ -θTΓθ-1θ -trWTΓW-1W-tr(VTΓV-1V)

Design the input τ

τ=e+Yθ+WTσ+βδsgnr+βrr

V=-eTαe+rTYθ+rTWTσ-rTβδsgnr-rTβrr-rTWTσ'Φ-rTδ-θTΓθ-1θ -trWTΓW-1W-tr(VTΓV-1V)

Design for θ, we want to get,

rTYθ-θTΓθ-1θ=0

θTΓθ-1θ=rTYθ

So,

θ=ΓθYTr

Design for W, we want to get,
rTWTσ-trWTΓW-1W=0

We know,

trbaT= aTb

trWTσrT=tr(WTΓW-1W)

So,

W=proj(ΓWσrT)

Design for V, we want to get,

rTWTσ'Vζ-trVTΓV-1V=0

We know,

trbaT= aTb

trVζrTWTσ'=trVTΓV-1V

So,

V=proj(ΓVζrTWTσ')

On substitution we get,

Vly≤ -eTαe-rTβrr

Using Barbalat’s Lemma we can show the e,ryields 0 which implies Asymptotic tracking

SIMULATION AND RESULTS –

(a) Selection of basis function -

For single-layer neural network, a polynomial basis was used to estimate the unstructured unknown τd since it was able to give a better estimate when compared to the Gaussian basis with uniformly distributed centers. And for two-layer neural network, Gaussian basis with uniformly distributed centers was used to estimate the unstructured unknown τd.

(b) Norm of tracking error and filtered tracking error

For Robust controller

For single-layer neural network

For two-layer neural network

Analysis –

The robust controller did not perform very well and had significantly large errors as indicated by the RMS filter tracking error norm of 1.32 and RMS error norm of 0.39. The single layer performed better that robust controller and brought the RMS filter tracking error to 0.7 and RMS error norm of 0.2. The two-layer performed a little better that single-layer controller in bringing down the RMS filter tracking error to 0.69 and RMS error norm remained the same at 0.2. The two-layer performed better overall.

(c) Function Approximation Error

For Robust Controller

For single-layer neural network

For two-layer neural network

Analysis –

Two-layer neural network performed the best in approximating the function with a RMS difference in function estimation of 4.97. The single-layer neural network was able did moderately well with a RMS difference in function estimation of 8.22, performing twice as better than the robust controller which has an RMS difference in function estimation of 21.9.

(d) Input Plots

For Robust Controller

Plot of total input

For Single-layer Neural Network

For Two-layer Neural Network

Analysis –

The robust controller relied mainly on the feedback terms of the input as seen in the plot above where the feedback curve in blue is fluctuating more compared to the neural networks. Whereas the single-layer and two-layer Neural Network relied mainly on the feedforward part of the input as indicated by the almost horizontal blue curve of feedback part of the input.

Two-layer neural network relied on the feedback part the least.